BIRDWATCH CAM  ·  FLASHING THE FIRMWARE

Arduino IDE Settings

DFRobot ESP32-S3 AI CAM (DFR1154) — the exact Tools-menu configuration used to build and flash the camera.

BEFORE YOU START  ·  INSTALL THESE ONCE

1.  Add the board package URL. Arduino IDE → Settings / Preferences → Additional boards manager URLs, paste:
https://espressif.github.io/arduino-esp32/package_esp32_index.json

2.  Install the board core. Tools → Board → Boards Manager → search "esp32" → install "esp32 by Espressif" (version 3.x).

3.  Install the library. Tools → Manage Libraries → search "ArduinoJson" → install (version 7.x).

TOOLS MENU  ·  SET EACH OF THESE

| Board | ESP32S3 Dev Module | Selects the ESP32-S3 chip on the DFR1154. (Under Tools → Board → esp32.) |
| --- | --- | --- |
| USB CDC On Boot | Enabled | Makes the board appear as a serial port so uploads and Serial Monitor work over USB-C. |
| Flash Size | 16MB (128Mb) | The DFR1154 has 16 MB of flash; the wrong size makes uploads or boot fail. |
| PSRAM | OPI PSRAM | Turns on the 8 MB PSRAM the camera needs for image buffers — without it the camera won't initialize. |
| Partition Scheme | Huge APP (3MB No OTA / 1MB SPIFFS) | The firmware needs the 3 MB app partition; the default (~1.3 MB) is too small. |
| Port | /dev/cu.usbmodem… · COM… | The port that appears when you plug the camera in. Mac: cu.usbmodem…; Windows: COM3+. |

Every other Tools option can stay at its default (Upload Speed 921600, CPU 240 MHz, Core Debug Level None).

UPLOAD & FIRST BOOT

›  Open esp32aicam/esp32aicam.ino and click Upload (the → arrow).

›  Port missing, or upload fails?  Hold the board's BOOT button while plugging in USB, then upload.

›  If it doesn't start after upload, tap the RST (reset) button once.

›  Open Serial Monitor at 115200 baud — it prints the camera's IP address, e.g. http://192.168.1.182/

›  First boot with no Wi-Fi saved: the camera makes an AI-Camera-Setup hotspot — join it, open 192.168.4.1, enter your Wi-Fi.

✓ VERIFIED    These are the settings this camera was built and flashed with — a correct build reports about 1.29 MB used, ~41% of the 3 MB partition.